Enterprise Backup & Disaster Recovery (BDR) Plan

Document Reference: BDR-OPS-003

Effective Date: 11 September 2026

Scope: Network devices (Router1, Switch0), Enterprise Server0 (Web/DNS), endpoints, and inter-VLAN infrastructure.

1. Objectives and Resilience Targets

To prevent catastrophic outages and configuration data loss, critical operations are bounded by specific recovery metrics:

- Recovery Point Objective (RPO): Maximum acceptable data loss window.

  - Network Device Configurations (Router/Switch): RPO = 24 hours (or immediately following any change).

  - Application & Web Data (Server0): RPO = 12 hours.

- Recovery Time Objective (RTO): Maximum allowable downtime to restore full operations.

  - Core Routing & Gateway Services: RTO < 1 hour.

  - Critical Enterprise Web/DNS Services: RTO < 2 hours.

  - Departmental Endpoints & Peripherals: RTO < 4 hours.

2. Backup Strategy & Scheduling

The organization enforces the 3-2-1 Backup Rule: 3 total copies of data, across 2 different media formats, with at least 1 copy off-site.

| Asset | Backup Type | Target / Repository | Frequency | Retention |
| --- | --- | --- | --- | --- |
| Router1 & Switch0 | running-config & startup-config exports via TFTP/SSH | Secure Configuration Archive (TFTP / Central Git) | Daily automated export; mandatory manual trigger before/after changes | 90 days of versioned commits |
| Server0 (Web & DNS) | Full image & incremental site/DNS snapshots | Encrypted Network Attached Storage (NAS) / Off-site Cloud | Daily differential; weekly full image | 30 days daily, 12 months monthly |
| Workstation Profiles | Standardized OS Golden Images & redirected user folders | Central Server Storage | Monthly image refresh; real-time folder sync | 30 days version history |

2.1 Configuration Archival Commands (Network Devices)

To ensure running configurations are not lost during unexpected hardware resets or power loss, administrators must save configurations to NVRAM and export copies off-device:

Plaintext

! Step 1: Commit volatile RAM to non-volatile NVRAM

Router# copy running-config startup-config

! Step 2: Push remote backup to central TFTP repository

Router# copy startup-config tftp:

Address or name of remote host []? 192.168.10.10

Destination filename [Router1-confg]? Router1-cfg-backup-2026-10-05

3. Disaster Scenarios & Recovery Procedures

Scenario A: Network Device Configuration Corruption or Unsaved Reboots

- Risk: Loss of subinterfaces (Gig0/0.10–.40), trunk links, and access VLANs due to unexpected reboot before saving.

- Triage & Recovery:

  - Verify link and interface state: show ip interface brief and show vlan brief.

  - If missing, restore directly from the latest verified TFTP configuration:

Plaintext

Router# copy tftp: running-config

Address or name of remote host []? 192.168.10.10

Source filename []? Router1-cfg-backup-latest

  - Validate gateway reachability across all VLANs (192.168.10.1, 192.168.20.1, 192.168.30.1, 192.168.40.1).

  - Immediately execute write memory on both Router1 and Switch0.

Scenario B: Physical Core Switch / Router Failure

- Risk: Physical hardware burnout, port failure on trunk interfaces, or catastrophic hardware drop.

- Triage & Recovery:

  - Deploy pre-staged cold-standby hardware matching chassis specifications (e.g., Cisco 2911 / Catalyst 2960).

  - Patch physical cabling into identical port mappings (Fa0/1 = Server, Fa0/2-3 = Printers, Fa0/4-5 = APs, Fa0/6-24 = Workstations, Gig0/1 = Trunk to Router).

  - Load the baseline configuration file via console cable directly into global configuration mode.

  - Verify STP convergence by observing link LEDs turning solid green.

Scenario C: Server0 Data Loss or Web/DNS Service Crash

- Risk: Unavailability of internal web services ([www.google.com](https://www.google.com) mock / enterprise intranet) and localized domain name resolution.

- Triage & Recovery:

  - Check HTTP and DNS service daemon toggles under Server0 → Services.

  - If data is corrupted, restore the site directory (index.html) and DNS mapping tables from the last known good configuration image.

  - Clear client local DNS caches from workstation command prompts (ipconfig /flushdns) to purge invalid resolution records.

4. Testing, Auditing & Maintenance

- Quarterly Simulation Drills: Conduct quarterly mock failover drills simulating router configuration wipes and cold-start switch replacements to confirm compliance with target RTOs.

- Configuration Integrity Checks: Perform automated monthly diff-checks between active running configurations and the central backup repository to detect unauthorized changes or unsaved configuration drifts.

- Plan Maintenance: Review and re-sign this disaster recovery document annually or whenever major architectural changes (e.g., adding dynamic routing protocols, new VLAN segments, or redundant firewall failovers) are introduced.