Enterprise Network Infrastructure Troubleshooting Guide

Project: Multi-VLAN Corporate Network & Services Implementation (Packet Tracer)

Document Reference: KB-NET-TS-001

Scope: Diagnosis, root cause analysis, and resolution procedures for VLAN segmentation, inter-VLAN routing, wireless connectivity, and intranet service accessibility.

1. Network Architecture Reference

The topology implements a Router-on-a-Stick model connecting a central Layer 2 switch (Switch0), an edge router (Router1), access points (AccessPoint0, AccessPoint1), enterprise servers, and client devices.

| VLAN ID | Segment Name | Subnet Network | Default Gateway | Allocated Switch Ports | Key Devices |
| --- | --- | --- | --- | --- | --- |
| 10 | Servers | 192.168.10.0/24 | 192.168.10.1 | Fa0/1 | Server0 (192.168.10.10) |
| 20 | Workstations / Departments | 192.168.20.0/24 | 192.168.20.1 | Fa0/6 – Fa0/24 | Software Dev, Finance, Marketing, HR PCs |
| 30 | Printers | 192.168.30.0/24 | 192.168.30.1 | Fa0/2 – Fa0/3 | Printer0, Printer1 |
| 40 | Wireless / Management | 192.168.40.0/24 | 192.168.40.1 | Fa0/4 – Fa0/5 | AccessPoint0, AccessPoint1, Laptop0-2 |
| Trunk | Router Link | N/A (802.1Q) | N/A | Gig0/1 (Switch) → Gig0/0 (Router) | Subinterfaces: Gig0/0.10, .20, .30, .40 |

2. Issues Log, Root Cause Analysis & Resolutions

Incident 1: Total Inter-VLAN Routing Failure & 100% ICMP Packet Loss

- Symptoms:

  - Pings from wireless laptops (Laptop0, Laptop2) to their default gateway 192.168.40.1 resulted in Request timed out (100% loss).

  - Workstation PC0(12) could successfully ping local neighbors in VLAN 20 (192.168.20.4), but was unable to reach its default gateway 192.168.20.1 or the server 192.168.10.10.

- Root Cause Analysis:

  - The physical interface GigabitEthernet0/0 on Router1 was administratively down, showing red link indicators on the trunk connection.

  - Sub interface configurations on Router1 were not saved to NVRAM prior to an interface flap/reset, reverting all sub interfaces (.10, .20, .30, .40) to unassigned states.

- Resolution Steps:

  - Brought up the parent physical interface on Router1:

Plaintext

Router(config)# interface GigabitEthernet0/0

Router(config-if)# no shutdown

  - Rebuilt 802.1Q encapsulation subinterfaces and applied gateways:

Plaintext

Router(config)# interface GigabitEthernet0/0.10

Router(config-subif)# encapsulation dot1Q 10

Router(config-subif)# ip address 192.168.10.1 255.255.255.0

Router(config-subif)# no shutdown

Router(config)# interface GigabitEthernet0/0.20

Router(config-subif)# encapsulation dot1Q 20

Router(config-subif)# ip address 192.168.20.1 255.255.255.0

Router(config-subif)# no shutdown

Router(config)# interface GigabitEthernet0/0.30

Router(config-subif)# encapsulation dot1Q 30

Router(config-subif)# ip address 192.168.30.1 255.255.255.0

Router(config-subif)# no shutdown

Router(config)# interface GigabitEthernet0/0.40

Router(config-subif)# encapsulation dot1Q 40

Router(config-subif)# ip address 192.168.40.1 255.255.255.0

Router(config-subif)# no shutdown

  - Committed running configuration to persistent storage:

Plaintext

Router# copy running-config startup-config

- Verification: show ip interface brief confirms all subinterfaces are Status: up and Protocol: up.

Incident 2: Layer 2 Switch Port Misassignment & VLAN Isolation

- Symptoms:

  - Client endpoints could communicate within unmanaged default segments, but isolated servers and peripheral printers could not receive packets from departmental workstations.

- Root Cause Analysis:

  - Execution of show vlan brief on Switch0 revealed that Fa0/1 (Server) and Fa0/2 - Fa0/3 (Printers) were residing in default VLAN 1, leaving VLAN 10 and VLAN 30 with zero active access ports.

- Resolution Steps:

  - Reassigned dedicated switch ports to their designated access VLANs:

Plaintext

Switch(config)# interface FastEthernet0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

Switch(config)# interface range FastEthernet0/2 - 3

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 30

Switch(config)# interface range FastEthernet0/6 - 24

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 20

- Verification: show vlan brief confirms Fa0/1 assigned to 10 Server, Fa0/2-3 assigned to 30 Printers, and Fa0/6-24 assigned to 20 Workstations.

Incident 3: Missing Client Default Gateways (Layer 3 Routing Dropped)

- Symptoms:

  - Wireless laptops showed IP address 192.168.40.4 or 192.168.40.2, but pings destined outside the subnet failed immediately.

- Root Cause Analysis:

  - Client ipconfig output revealed Default Gateway: 0.0.0.0. While Layer 2 frames could reach the local switch, the TCP/IP stack discarded egress packets intended for remote subnets.

- Resolution Steps:

  - Configured static gateway values across wireless clients:

    - IPv4 Address: 192.168.40.2

    - Subnet Mask: 255.255.255.0

    - Default Gateway: 192.168.40.1

- Verification: ipconfig outputs valid non-zero gateway configuration; ping to gateway 192.168.40.1 succeeds.

Incident 4: Simulation Execution Freeze

- Symptoms:

  - Packets appeared stalled in buffer; ipconfig and physical link states appeared green, yet ICMP requests consistently timed out.

- Root Cause Analysis:

  - Packet Tracer was toggled into Simulation Mode, halting autonomous packet forwarding until manual simulation triggers were stepped through.

- Resolution Steps:

  - Toggled environment state to Realtime Mode (Alt + R).

  - Applied Fast Forward Time (>>) to bypass 30-second Spanning Tree Protocol (STP) listening/learning delays and flush initial ARP resolution cycles.

- Verification: Simulation tray indicators switched to Successful for client-to-gateway and client-to-server PDUs.

3. Standard Verification Checklist

When diagnosing future connectivity faults across this environment, follow this structured troubleshooting flow:

- Physical & Link Layer:

  - Verify link lights: Green = Active; Amber = STP Listening/Learning; Red = Port administratively down or disconnected.

  - Confirm trunk link carrying 802.1Q tags: show interfaces trunk on switch.

- Switch VLAN Assignment:

  - Verify port-to-VLAN mappings: show vlan brief.

- Router Routing & Interface State:

  - Verify subinterface status: show ip interface brief.

  - Verify routing table: show ip route.

- Client IP Stack:

  - Verify local IP, subnet mask, and default gateway: ipconfig / ipconfig /all.

- Progressive Ping Sequence:

  - Ping 127.0.0.1 (TCP/IP stack verification).

  - Ping client static IP (NIC verification).

  - Ping local Default Gateway (Layer 2 link and router subinterface verification).

  - Ping remote gateway/server (Inter-VLAN routing verification).