Enterprise Risk Assessment Matrix

Document Reference: RAM-SEC-006

Effective Date: 11 September 2026

Scope: Multi-VLAN network infrastructure, routing topology, core server services, and enterprise endpoints.

1. Risk Scoring Methodology

Risk is determined by calculating the product of Likelihood and Impact:

Likelihood Scale (1 to 5)

- Rare: Highly unlikely to occur; requires complex prerequisites.

- Unlikely: Possible, but not expected during normal operations.

- Moderate: Has occurred occasionally or is moderately easy to exploit.

- Likely: Expected to happen multiple times per year without controls.

- Almost Certain: Continual operational hazard; inevitable without mitigation.

Impact Scale (1 to 5)

- Negligible: Minor inconvenience; no service downtime or data leakage.

- Minor: Localized single-device outage; minimal resolution effort.

- Moderate: Departmental outage or minor security breach; business operations degraded.

- Major: Subnet-wide failure, severe data compromise, or loss of core services.

- Catastrophic: Total infrastructure shutdown, enterprise data loss, or regulatory non-compliance.

Heat Map & Action Levels

| Likelihood \ Impact | 1 - Negligible | 2 - Minor | 3 - Moderate | 4 - Major | 5 - Catastrophic |
| --- | --- | --- | --- | --- | --- |
| 5 - Almost Certain | Low (5) | Medium (10) | High (15) | Critical (20) | Critical (25) |
| 4 - Likely | Low (4) | Medium (8) | High (12) | High (16) | Critical (20) |
| 3 - Moderate | Low (3) | Medium (6) | Medium (9) | High (12) | High (15) |
| 2 - Unlikely | Low (2) | Low (4) | Medium (6) | Medium (8) | High (10) |
| 1 - Rare | Low (1) | Low (2) | Low (3) | Low (4) | Medium (5) |

- Critical (20–25): Immediate executive mitigation required; daily tracking.

- High (12–16): Remediation required within 14 days; active senior monitoring.

- Medium (6–10): Routine controls required; reviewed monthly.

- Low (1–5): Acceptable risk under standard operational procedures.

2. Infrastructure Risk Register

| Risk ID | Threat & Vulnerability Description | Pre-Score (L × I) | Mitigations & Implemented Safeguards | Post-Score (L × I) | Owner |
| --- | --- | --- | --- | --- | --- |
| RSK-01 | Unsaved Running Configurations on Network Hardware Router or switch reboots/power loss wipes dynamic subinterfaces and trunk links, halting cross-VLAN communication. | (High) | Enforce standard operating procedure requiring copy run start after any modification. Automated daily TFTP configuration archiving. | (Low) | Network Lead |
| RSK-02 | Physical Trunk Link Failure or Shutdown Accidental physical disconnect or administrative shutdown on Gig0/0 severs all routing between workstations, wireless clients, and servers. | (High) | Implementation of port monitoring, spanning-tree verification, and pre-staged cold standby routing hardware. | (Low) | Network Lead |
| RSK-03 | Access Point Port-VLAN Misconfiguration Connecting APs to unmanaged or incorrect switch ports causes unauthorized VLAN cross-talk or isolated wireless clients. | (High) | Explicit port assignment on switch interfaces (Fa0/4–Fa0/5 to VLAN 40), BPDU Guard, and disabling unused physical ports. | (Low) | Systems Admin |
| RSK-04 | Lateral Movement via Unrestricted Inter-VLAN Routing Compromised endpoint in Department/Workstations (VLAN 20) scans and pivots directly into Server0 (VLAN 10). | (High) | Application of inbound access control lists (ACLs) on router subinterfaces (Gig0/0.20), restricting traffic to HTTP (port 80) and DNS (port 53). | (Low) | Security Team |
| RSK-05 | Core Service (DNS/Web) Outage on Server0 DNS daemon crash or web directory corruption halts enterprise intranet access across all departments. | (High) | Regular system snapshots, automated health daemon checks, local hosts file fallback, and disaster recovery image snapshots. | (Low) | Systems Admin |
| RSK-06 | Rogue DHCP Server / Gateway Spoofing Unauthorized client starts broadcasting IP assignments, distributing bogus default gateways and causing a localized denial of service. | (Medium) | Implementation of DHCP Snooping on Switch0, setting all departmental access ports to untrusted and router trunk link to trusted. | (Low) | Network Lead |
| RSK-07 | Endpoint Privilege Escalation & Unapproved Software Workstation users executing malicious scripts or installing shadow IT utilities without admin clearance. | (High) | Standard user role enforcement, elimination of root desktop logins, Polkit authentication prompts, and automated patch management. | (Low) | IT Support Lead |

3. Continuous Monitoring & Review Cadence

- Monthly Vulnerability Sweeps: Conduct internal scans across subnets 192.168.10.0/24, 192.168.20.0/24, and 192.168.40.0/24 to confirm no new unauthorized ports or listening daemons are exposed.

- Quarterly Audit of ACLs & Switch Ports: Compare active switch port allocations (show vlan brief) against approved network documentation to identify orphan or misassigned ports.

- Annual Risk Register Re-certification: Formal evaluation of this risk matrix conducted annually or immediately following any network redesign.